## ENOLA Uses Embench

### Measurement

## Using Embench, ENOLA measures:

## Runtime overhead

## Extra cycles added due to instrumentation

## Code size overhead

## Increase in binary size due to ENOLA logic

## Attestation data size

## Size of TO, Mf, Mb

## . ENOLA High-Level Design

ENOLA introduces a new authenticator structure:

### Authenticator = (TO , Mf , Mb)

- TO (Occurrence Trace)

  - Records how many times each basic block executes

  - NOT a full execution order

  - Size = O(|V|)

- Mf (Forward Measurement)

  - Chained MAC over branch targets

- Mb (Backward Measurement)

  - Chained MAC over function returns

➡️ Achieves linear space complexity with strong security.

## 2. Key Observation

- Recording full execution traces is unnecessary and expensive.

- Software hash functions are too slow for MCUs.

- Modern MCUs already provide:

  - Trusted Execution Environments (TEE),

  - Hardware-assisted authentication (ARM Pointer Authentication).

➡️ ENOLA exploits these hardware features to redesign CFA.

## 5. System & Threat Model

- MCU supports:

  - TrustZone (TEE),

  - ARM Pointer Authentication (PAC).

## 12. Evaluation Flow

Evaluated on:

- Syringe pump (real CPS app),

- Embench suite,

- wolfSSL (large real-world codebase).

Metrics:

- Execution overhead,

- Code size overhead,

- Authenticator size.

➡️ ENOLA:

- Produces small, linear-size attestation data,

- Runs 100× faster than software-hash CFA,

- Outperforms or matches state-of-the-art CFA schemes.

# How ENOLA Works (Step-by-Step)

ENOLA is a Control-Flow Attestation (CFA) system that allows a remote verifier to confirm that an embedded program executed exactly along an intended control-flow path, even if the device may be compromised at runtime.

## High-Level Idea

Instead of recording the entire execution trace, ENOLA:

- Records how many times each basic block executes.

- Cryptographically authenticates control-flow transfers using hardware support.

- Keeps all security-critical data out of memory.

This makes ENOLA fast, scalable, and secure for low-end embedded systems.

## ENOLA Architecture Overview

ENOLA works in three phases:

- Compile-time

- Run-time (on the device)

- Verification-time (remote verifier)

## 1. Compile-Time: Preparing the Program

### (a) Program Instrumentation (LLVM Compiler)

The ENOLA compiler automatically inserts special instructions into the program:

#### Forward Control-Flow (branches, calls)

- At the start of each branch target basic block:

  - Update a chained measurement (Mf) using a PAC instruction.

  - Report the basic block execution to the secure world.

#### Backward Control-Flow (returns)

- Before every function return:

  - Update a second chained measurement (Mb) using the return address.

### (b) Hardware-Based Measurements

- Uses ARM Pointer Authentication (PAC) (pacg) instruction.

- Each new measurement depends on:

  - Previous measurement value,

  - Current control-flow target.

- This forms a cryptographic chain.

### (c) Register Reservation

- Two CPU registers are permanently reserved:

  - One for Mf (forward path),

  - One for Mb (backward path).

- These registers are:

  - Never spilled to memory,

  - Never accessible by untrusted code.

➡️ This prevents memory corruption attacks.

### (d) Code Scanner & Analyzer

- Code scanner ensures:

  - No instruction can overwrite PAC keys or measurement registers.

- Analyzer builds a list of valid indirect jump targets.

## 2. Run-Time: Attestation During Execution

### (a) Secure Initialization

- Device boots.

- ENOLA attestation engine runs in TEE (TrustZone).

- Receives a nonce from the verifier.

- Loads secret keys into PAC key registers.

### (b) Program Execution (Normal World)

- Program executes normally.

- Instrumented code:

  - Updates Mf on each branch,

  - Updates Mb on each return,

  - Reports basic block execution counts to the secure world.

### (c) Trace Collection

Instead of storing full traces, ENOLA records:

Basic block start address → execution count

This is called the Occurrence Trace (TO).

➡️ Space complexity = O(number of basic blocks).

### (d) Attestation Report Generation

After program completion:

- Secure world collects:

  - TO (block counts),

  - Mf (forward measurement),

  - Mb (backward measurement).

- Signs them with a secret key and nonce.

- Sends the report to the verifier.

## 3. Verification-Time: Checking Correct Execution

### (a) Report Validation

The verifier:

- Verifies the signature,

- Checks freshness via nonce,

- Rejects illegal jump targets.

### (b) Abstract Execution with Backtracking

Using the program’s CFG, the verifier:

- Starts from the entry block.

- Follows possible branches guided by TO counts.

- Recomputes:

  - Forward measurement (Mf),

  - Backward measurement (Mb).

- Backtracks when paths do not match TO.

### (c) Acceptance Condition

The execution is valid only if:

- All TO counts are consumed correctly,

- Recomputed Mf and Mb match the reported values.

Otherwise → control-flow attack detected.

## Why ENOLA Is Efficient

| Aspect | Traditional CFA | ENOLA |
| --- | --- | --- |
| Trace size | O(execution length) | O(#basic blocks) |
| Cryptography | Software hash | Hardware PAC |
| Measurement storage | Memory | CPU registers |
| MCU support | Limited | Native |

## Why ENOLA Is Secure

- Hardware-enforced cryptography

- No secret in memory

- TEE isolation

- Chained measurements

- ROP and hijacking detected

## One-Sentence Explanation (Viva Ready)

ENOLA works by instrumenting programs to record basic-block execution counts and hardware-authenticated control-flow measurements, enabling a remote verifier to reconstruct and verify the exact execution path efficiently on low-end embedded devices.

If you want, I can also:

- Explain ENOLA with a diagram,

- Compare ENOLA vs C-FLAT / OAT / BLAST,

- Give a short 5-line exam answer,

- Or explain it purely from a security attack perspective.