## 4️⃣ Area Overhead Evaluation (Hardware Cost)

### FPGA Resource Usage

On Virtex-7 XC7Z020:

| Resource | Usage |
| --- | --- |
| Registers | ~4% |
| LUTs | ~6% |
| BRAMs | 49 × 36Kb |

This corresponds to:

- ~20% additional logic overhead over the base Pulpino SoC

WORK FLOW:

Below is a step-by-step explanation of the LO-FAT flow, from program execution to verifier decision. I’ll keep it conceptual first, then map it to hardware blocks, which is usually what examiners and reviewers expect.

## 1️⃣ Pre-Execution (Verifier Side – Offline)

Before the device ever runs:

- Verifier (V) has the program binary S.

- V performs offline analysis to build:

  - The Control Flow Graph (CFG) of S

  - Expected loop structures and valid paths

- V stores this CFG as the reference model.

➡️ This step is done once, not during attestation.

## 2️⃣ Attestation Request (Start of Protocol)

- Verifier sends to Prover (P):

  - Program identifier id_S

  - Program input i

  - Random nonce N (for freshness)

- Prover:

  - Has already passed static attestation (binary is intact)

  - Starts executing program S(i)

## 3️⃣ Runtime Execution with LO-FAT (Core Flow)

This is where LO-FAT operates in parallel with the CPU.

### 🔹 Step 3.1: Instruction Execution

- CPU executes instructions normally

- No stalls, no instrumentation, no ISA changes

LO-FAT only observes execution.

### 🔹 Step 3.2: Branch Interception (Branch Filter)

For every clock cycle:

- Branch Filter reads:

  - Current PC

  - Executed instruction type

- If instruction is:

  - Branch

  - Jump

  - Call

  - Return

➡️ Extract (Src, Dest) address pair

- Decide:

  - Loop branch?

  - Non-loop branch?

### 🔹 Step 3.3: Non-Loop Branch Handling

If branch is not inside a loop:

- (Src, Dest) is sent directly to:

  - Hash Engine

- Hash engine updates the cumulative hash A

This records straight-line control flow.

### 🔹 Step 3.4: Loop Detection (No Instrumentation!)

LO-FAT detects loops dynamically using hardware heuristics:

- Backward branch without link register update ⇒ loop

- Loop entry = branch target

- Loop exit = instruction after branch

This works on:

- RISC-V

- ARM

- PowerPC-like architectures

## 4️⃣ Loop Execution Handling (Key Innovation)

Instead of hashing every iteration, LO-FAT compresses loop behavior.

### 🔹 Step 4.1: Loop Path Encoding

Inside a loop:

- Each conditional branch produces:

  - Taken → 1

  - Not taken → 0

- Loop Monitor builds a Path_ID:

  - Unique bit-string per loop path

Example:
if-else loop:

Path 1 → 0011

Path 2 → 011

### 🔹 Step 4.2: First Time a Loop Path Appears

If Path_ID is new:

- Loop Monitor:

  - Signals hash engine

- Hash engine hashes all (Src, Dest) pairs for that loop path once

- Loop counter for that path is initialized

### 🔹 Step 4.3: Repeated Loop Iterations

If same Path_ID repeats:

- ❌ No hashing

- ✅ Just increment loop counter

This avoids exponential hash growth.

### 🔹 Step 4.4: Indirect Branches in Loops

For function pointers / returns:

- Targets are encoded into small IDs (n-bit)

- Stored in CAM-based register files

- If target exceeds limit:

  - Special “overflow” encoding sent to verifier

## 5️⃣ Loop Exit and Metadata Generation

When loop exits:

- Loop Monitor signals loop end

- Metadata Generator creates L:

  - Executed Path_IDs

  - Order of first occurrence

  - Iteration counts

  - Indirect branch targets

Metadata L is stored securely on-chip.

## 6️⃣ Program Completion

At program exit:

- Final hash A is ready

Prover computes:
R = Sign(A || L || N)

- Signature key is hardware-protected

## 7️⃣ Attestation Verification (Verifier Side)

Verifier performs:

- Signature verification (authenticity)

- Nonce check (freshness)

- Reconstructs control flow using:

  - CFG

  - Hash A

  - Loop metadata L

- Checks:

  - Is this a valid CFG path?

  - Do loop counts match?

  - Are indirect targets legal?

## 8️⃣ Attack Detection Capability

LO-FAT detects:

| Attack Type | Detected? | Why |
| --- | --- | --- |
| ROP / JOP | ✅ | Invalid (Src, Dest) |
| Loop counter attacks | ✅ | Iteration mismatch |
| Non-control data attacks | ✅ | Control flow deviation |
| Pure data attacks | ❌ | No control-flow impact |

## 🔁 One-Line Flow Summary (Exam Gold)

LO-FAT observes every control-flow instruction in hardware, hashes non-loop branches directly, compresses loop behavior using path encodings and counters, signs the execution trace, and allows a remote verifier to validate the exact runtime control flow against a known CFG—without software instrumentation or performance overhead.

If you want next:

- Sequence diagram

- Comparison with C-FLAT flow

- Explain LO-FAT in 5-minute viva style

- Map flow to Fig. 3 (block-by-block explanation)